| Task ID | Task Name | Description |
| --- | --- | --- |
| 0.1 | Literature Review I | Read 2 papers on D especially D Lite for mobile robots. Understand the core idea. |
| 0.2 | Literature Review II | Watch a YouTube video explaining Model Predictive Control (MPC) at a high level. |
| 0.3 | Literature Review III | Read a short article/tutorial on B-splines and why they are good for smooth paths. |
| 0.4 | Tool Setup I | Install Ubuntu 22.04 if not already done. Verify installation. |
| 0.5 | Tool Setup II | Follow the official ROS2 Humble installation guide. Install it. |
| 0.6 | Tool Setup III | Run ros2 --help and create a dummy workspace. Build it. Just hello world. |
| 0.7 | Tool Setup IV | Install Gazebo Garden. Launch it from terminal to see the empty world. |
| 1.1 | PX4 Setup I | Clone the PX4-Autopilot repo using the master branch for Garden compatibility. |
| 1.2 | PX4 Setup II | Run make px4_sitl gazebo-classic yes, classic works with Garden to see the Iris drone in empty world. |
| 1.3 | QGControl Test | Install QGroundControl. Connect it to the running SITL simulation. See the HUD. |
| 1.4 | ROS2-PX4 Bridge | Find and build the px4roscom package. Build it in your workspace. |
| 1.5 | Topic Discovery | Run ros2 topic list while sim is running. Identify fmuout_vehicle_odometry and fmuin_vehicle_control_mode. |
| 1.6 | Basic Publisher | Write a simple Python node publishing a single dummy message to a test topic. |
| 1.7 | World Building I | Create a simple Gazebo world with a ground plane and a few static boxes obstacles. Save as .sdf. |
| 1.8 | World Building II | Launch your custom world with the PX4 drone in it. |
| 2.1 | Map Representation | Create a Python class GridMap3D. Initialize a 3D numpy array representing free space (0). |
| 2.2 | Add Obstacles | Add a method to add obstacles by changing a cell to occupied (1). Manually add a few. |
| 2.3 | D Node Class | Create a Node class with g, rhs, key, pos (x, y, z), and backpointer attributes. |
| 2.4 | D Core I - Initialize | Create a DStarLite class. Write init method to set start, goal, and grid map. |
| 2.5 | D Core II - Key Calc | Write the calculate_key_node method. |
| 2.6 | D Core III - Get SuccPred | Write get_successors_pos and get_predecessors_pos methods. |
| 2.7 | D Core IV - Cost Func | Write c(nodeA, nodeB) to return cost 1 for free, infinity for obstacle. |
| 2.8 | D Core V - Update Vertex | Write update_vertex(u) method. |
| 2.9 | D Core VI - Main Loop | Write compute_shortest_path method. |
| 2.10 | Test in Isolation | Test DStarLite on tiny 2x2x2 grid without ROS. Print path. Debug until it works. |
| 2.11 | Integrate with World | Write a node reading world obstacle positions and initializing GridMap3D. |
| 2.12 | Publish Path | Modify D node to publish computed path as nav_msgs/msg/Path message. |
| 2.13 | Visualize in RVIZ2 | Launch RVIZ2, add display for Path message, see raw D path in 3D. |
| 3.1 | Install Library | Find and install a Python B-spline library (e.g., scipy.interpolate.BSpline, splipy). |
| 3.2 | Smoother Node | Create new node subscribing to raw D Path message. |
| 3.3 | Extract Path Points | In callback, extract list of (x, y, z) points from Path message. |
| 3.4 | Fit B-spline | Write code to fit 3D B-spline curve through path points with default config. |
| 3.5 | Sample Smooth Path | Sample 100 points from smooth B-spline curve to create new dense path. |
| 3.6 | Publish Smooth Path | Publish new smooth path as nav_msgs/msg/Path message. |
| 3.7 | Visualize in RVIZ2 | View raw (red) and smooth (green) paths in RVIZ2. Tweak B-spline parameters. |
| 4.1 | MPC Theory | Read blog post about MPC optimization over receding horizon. |
| 4.2 | Choose MPC Tool | Research and choose Python MPC toolkit (do-mpc, acados). Install. |
| 4.3 | Drone Model I | Define simple linear/kinematic model for drone (e.g., double integrator x, y, z). |
| 4.4 | Drone Model II | Implement model within chosen MPC framework. |
| 4.5 | MPC Formulation I | Setup MPC optimizer with cost function minimizing error to reference path. |
| 4.6 | MPC Formulation II | Add input rate constraints for smooth control, state constraints to avoid underground. |
| 4.7 | MPC Node Skeleton | Create node for MPC. Subscribe to smooth Path and vehicle Odometry. |
| 4.8 | MPC Initialization | Initialize MPC controller and set reference trajectory from path. |
| 4.9 | MPC Control Loop | Write timer callback fetching odometry, running MPC solver, publishing first control input. |
| 4.10 | Publish Controls | Publish computed controls (thrust, pitch, roll) to PX4 ROS topic. |
| 4.11 | Test in Hover | Test MPC node for static point hover. Debug. |
| 5.1 | Static Flight Test | Run full stack on static world. Drone takes off, flies smooth pre-planned path, lands. |
| 5.2 | Obstacle Sensing | Add subscriber to simulated depth sensor topic (e.g., pointcloud). |
| 5.3 | Map Updater | Write function converting pointcloud data into obstacles in GridMap3D. |
| 5.4 | Replan Trigger | Create logic to trigger D replanning when new obstacle detected too close to path. |
| 5.5 | Full Adaptive Test | Test full adaptive loop (sense, replan, smooth, track) with dynamic obstacle. |
| 5.6 | Tuning I - Smoothness | Tune B-spline parameters for balance of smoothness and obstacle clearance. |
| 5.7 | Tuning II - MPC | Tune MPC prediction horizon and weights for stable, accurate tracking. |
| 5.8 | Performance Metrics | Write script to log path length, tracking error, computation time. Run on tests. |
| 6.1 | Outline Thesis | Create detailed outline with chapters and sections. |
| 6.2 | Write Abstract | Draft summary paragraph on problem, method, results. |
| 6.3 | Write Introduction | Write introduction chapter: background, problem statement, objectives. |
| 6.4 | Write Literature Review | Compile notes into literature review chapter. |
| 6.5 | Write Methodology | Document system architecture, explain D, MPC, B-spline. |
| 6.6 | Write Implementation Setup | Document software setup: ROS2, PX4, Gazebo, libraries. |
| 6.7 | Write Results I | Describe test scenarios, present results with visuals (RVIZ2 paths, graphs). |
| 6.8 | Write Results II | Analyze results: successes, limitations. |
| 6.9 | Write Conclusion | Summarize work, contributions, suggest future work. |
| 6.10 | Format Bibliography | Use Zotero or similar tool to manage references. |
| 6.11 | Final Proofread | Read entire thesis aloud to catch errors. Ask friend to review. |
| 6.12 | Submit! | Celebrate accomplishment of building complex system and writing thesis. |